| ITEM NUMBER: CM-01980 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| REV: |  | DESCRIPTION: | ECO: | DATE: | APPROVED: |
| 0.1 |  | SENT TO CUSTOMER FOR REVIEW |  | 09/09/2024 | K. FOSMARK |
| 0.2 |  | DESIGN UPDATES SENT TO CUSTOMER FOR REVIEW |  | 12/13/2024 | J. PARKER |
| 0.3 |  | UPDATES FOR INTERNAL INTERFACES, NEXT REVISION REQUIRES HEATER PINOUT FINALIZATION AND FEEDBACK FROM VENDOR |  | 5/13/2025 | J. PARKER |
| 0.4 |  | UPDATED CONNECTOR PINOUTS FOR J2, J5, AND J6 |  | 5/16/2025 | K. FOSMARK |
| 0.5 |  | UPDATED WITH HEATER AND THERMISTORS FOR OPTICAL ASSEMBLY, HEATER PARAMETERS CORRECTED |  | 6/17/2025 | K. FOSMARK |
| 0.6 |  | FINAL QUARTUS REVIEW PRIOR TO REV 1 RELEASE |  | 7/16/2025 | D. GOEL |

TABLE OF CONTENTS

1.0	INTRODUCTION	5

1.1.	PURPOSE	5

1.2.	SCOPE	5

1.3.	USAGE	5

1.4.	CONTROL AUTHORITY	5

1.5.	STANDARDS AND SPECIFICATIONS	5

2.0	APPLICABLE DOCUMENTS	5

2.1.	INTERNAL DOCUMENTATION	5

2.2.	EXTERNAL DOCUMENTATION	6

3.0	FARSIGHT SYSTEM OVERVIEW	6

3.1.	TURION MISSION AND COMPANY PROFILE	6

3.2.	FARSIGHT COMPLIANCE	6

4.0	EXTERNAL ELECTRICAL INTERFACE DEFINITIONS	7

4.1 BLOCK DIAGRAM	7

4.2	SIGNAL DEFINITIONS	8

4.2.1	POWER SUPPLY	8

4.2.2	RS422 INTERFACE	8

4.2.3	Pulse per Second (PPS) INTERFACE	8

4.2.4	10MHz INTERFACE	9

4.2.5	TRIG INTERFACE	9

4.2.6	INTERRUPT INTERFACES	9

4.2.7	GENERAL PURPOSE INTERFACES	10

4.2.8	PROGRAMMING INTERFACE	10

4.2.9	ETHERNET INTERFACES	11

4.2.10	PCIe INTERACE	11

4.2.11	HEATER AND THERMISTOR INTERFACE	12

4.3	CONNECTOR INTERFACE PINOUT DESCRIPTIONS	14

4.3.1	J1: TELEMETRY AND CONTROL	15

4.3.2	J2: PCIe SERIAL DATA	16

4.3.3	J3: Ethernet 1	17

4.3.4	J4: Ethernet 2	17

4.3.5	J5: POWER	18

4.3.6	QJ1 -QJ6: OTA HEATERS AND THERMISTORS	19

6.0	GROUNDING	21

7.0	INTERNAL ELECTRICAL INTERFACE DEFINITIONS	22

7.1	J6: Focusing Mechanism Interface	22

7.2	Focusing Mechanism Harness	22

7.3	J7: Sensor Board Interface	23

# LIST OF FIGURES

Figure 4-1: FARSIGHT Electrical Interface Block Diagram	7

Figure 7-1: Focusing mechanism harness illustration	23

# LIST OF TABLES

Table 2-1: Applicable TURION Documentation	5

Table 2-2: Applicable External Documentation	6

Table 4-1: Power Supply Interface Pins	8

Table 4-2: RS422 Interface Pins	8

Table 4-3: PPS Interface Pins	9

Table 4-4: 10 MHz Interface Pins	9

Table 4-5: Trig Interface Pins	9

Table 4-6: Interrupt Interface Pins	9

Table 4-7: General Purpose Interface Input Parameters	10

Table 4-8: General Purpose Interface Output Parameters	10

Table 4-9: Programming Interface Input Parameters	10

Table 4-10: Programming Interface Output Parameters	10

Table 4-11: Ethernet Interface Pins	11

Table 4-12: High-Speed PCIe Interface Pins	12

Table 4-13: High-Speed PCIe Interface Pins	12

Table 4-14: Focal Plane PCB Heater Interface Pins	12

Table 4-15: Focal Plane PCB Thermistor Interface Pins	13

Table 4-16: Optical Assembly Heater Interface Pins	13

Table 4-: Optical Assembly Thermistor Interface Pins	13

Table 4-14: Telemetry and Control Connector Pinout	15

Table 4-15: High-Speed Serial Data Connector Pinout	16

Table 4-16: Ethernet 1 Connector Pinout	17

Table 4-17: Ethernet 2 Connector Pinout	17

Table 4-18: Power Connector Pinout	18

Table 7-1: Focusing Mechanism Interface Pinout	22

Table 7-2: Sensor Board Interface Pinout	23

# LIST OF ACRONYMS

- ECO: Engineering Change Order

- FPGA: Field Programmable Gate Array

- GPI: General Purpose Input

- GPIO: General Purpose Input/Output

- GPO: General Purpose Output

- ICD: Interface Control Document

- LVCMOS: Low-Voltage Complementary Metal-Oxide-Semiconductor

- LVDS: Low-Voltage Differential Signaling

- PCB: Printed Circuit Board

- PPS: Pulse Per Second

- SERDES: Serializer/De-serializer

- TBA: To be Added

- TBD: To be Determined

- TBR: To be Revised

- INTRODUCTION

This document defines the interface functions and configuration parameters of the FARSIGHT electrical system as a payload of a DROID bus.

For Payload usage guidelines, operations, limitations, implementation, integration services, testing services, see the DROID.alpha Host Payload User Guide. The User Guide provides a complete description of how to implement a Payload System on TURION’s Hosted Payload Service.

  - PURPOSE

This document serves as an overview of the electrical system for the FARSIGHT telescope. This Interface Control Document (ICD) is not a standalone document and must be used in conjunction with the documents listed in section 2.0 Applicable Documents to ensure successful payload operations.

  - SCOPE

This document defines the electrical interfaces to the FARSIGHT imaging payload.

Definitions include:

- Control interface signal definitions, polarities, and levels are defined.

- Power supply voltage and current specifications.

- Heater and thermistor electrical interfaces.

- Connector pinout definitions

  - USAGE

This document is to be used by designers for the FARSIGHT payload electrical system to guide in the development and verification of FARSIGHT design’s conformance of integrating onto the DROID.Alpha bus.

  - CONTROL AUTHORITY

Upon release of version 1.0, this document is under the TURION quality control system and shall propose changes in accordance with the internal Engineering Change Order (ECO) process. Prior to version 1.0 this document is considered under a DRAFT status.

  - STANDARDS AND SPECIFICATIONS

The document adheres to multiple MIL and NASA standards and specifications, employing standard space industry nomenclature. The end users of this document are assumed to be Space Industry professionals.

- APPLICABLE DOCUMENTS

  - INTERNAL DOCUMENTATION

The following TURION documents and authorized changes form a part of this document.

Table 2-1: Applicable TURION Documentation

| Identifier | Document Title |
| --- | --- |
| CM-00945 | EXPORT CONTROL, GUIDELINES, DOCUMENT MARKING |

  - EXTERNAL DOCUMENTATION

TURION uses External Documents to implement industry standard practices to minimize payload integration complexity.

Table 2-2: Applicable External Documentation

| Authoring Entity | Identifier | Document Title |
| --- | --- | --- |
| NASA | NASA-HDBK-4001 | ELECTRICAL GROUNDING ARCHITECTURE FOR UNMANNED SPACECRAFT |

- FARSIGHT SYSTEM OVERVIEW

  - TURION MISSION AND COMPANY PROFILE

TURION was founded to accelerate the commercialization of space while focusing on key technological risks toward advancement of the greater vision of a prosperous and stable space economy. To address these risks, TURION uses a hardware-first approach to rapidly develop and iterate on space vehicles using a capability-based architecture to reduce risk and iteration cycle time.

  - FARSIGHT COMPLIANCE

FARSIGHT must adhere to the guidelines and requirements specified in this document and the DROID.Alpha Hosted Payload User Guide. This ensures successful integration and operation within the DROID.Alpha Satellite Bus Payload environment.

# EXTERNAL ELECTRICAL INTERFACE DEFINITIONS

## 4.1 BLOCK DIAGRAM

FARSIGHT interface illustrated in block diagram below (Reference Figure 41). The electrical interface is circled in red.

-

Figure 4-1: FARSIGHT Electrical Interface Block Diagram

## SIGNAL DEFINITIONS

This section defines the electrical interfaces to FARSIGHT. Characteristics of the power and control interfaces and communications protocols and standards used are defined. Connector pinout is defined in section 4.3.

### POWER SUPPLY

FARSIGHT requires one 28V power rail. All internal voltages are internally generated from this 28V rail. Power is supplied across PWR/PGND pins on the electrical interface. When power is within the Over/under voltage thresholds, FARSIGHT enters its lowest power operating mode and will respond to commands.

Table 4-1: Power Supply Interface Pins

| Power Supply Pins |  |
| --- | --- |
| Power Supply Positive | PWR |
| Power Supply Return ground | PGND |
| Power Supply Absolute Maximum Ratings |  |
| Voltage Range | -50V to 50V |
| Over/under voltage thresholds | 20V to 38V |
| Power Supply Recommended Operating Conditions |  |
| Voltage Range | 24V to 34V |
| Max current | 1.2A |
| Max power | 35W |
| Max inrush current | 2.5A |

### RS422 INTERFACE

FARSIGHT is primarily controlled using a full duplex RS422 interface running at 115.2 kbaud. The software ICD for FARSIGHT defines the protocol implemented.

The receive RS422 (RS422_BUS_TO_FARSIGHT_P/N) is terminated with a 100 ohm resistor.

Table 4-2: RS422 Interface Pins

| RS422 Pins |  |
| --- | --- |
| RS422 inputs to FARSIGHT | RS422_BUS_TO_FARSIGHT_P RS422_BUS_TO_FARSIGHT_ |
| RS422 outputs from FARSIGHT | RS422_FARSIGHT_TO_BUS_P RS422_FARSIGHT_TO_BUS_N |
| Electrical Signaling standard | ANSI/TIA/EIA-422-B |
| Receiver termination resistance | 100 ohm |
| Baud rate (default) | 115.2 kbaud |

### Pulse per Second (PPS) INTERFACE

Pulse per Second (PPS) is the one pulse per second timing reference from the bus. It is electrically using a differential Low-Voltage Differential Signaling (LVDS) receiver, including 100 ohm termination resistor. The rising edge (PPS_P going from low to high and PPS_N going from high to low) is the timing edge used for synchronization purposes.

Table 4-3: PPS Interface Pins

| PPS Pins |  |
| --- | --- |
| PPS inputs to FARSIGHT | PPS_P PPS_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Termination resistance | 100 ohm |
| Minimum PPS pulse width | 10 ms |

### 10MHz INTERFACE

10MHz is a reference clock from the bus (synchronized to PPS). It is electrically using a differential LVDS receiver, including 100 ohm termination resistor. Supply of 10MHz is optional and this port can be left open.

Table 4-4: 10 MHz Interface Pins

| 10MHz Pins |  |
| --- | --- |
| 10MHz inputs to FARSIGHT | 10MHZ_P 10MHZ_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Termination resistance | 100 ohm |

### TRIG INTERFACE

TRIG is a trigger input interface to FARSIGHT that can be used to trigger FARSIGHT to for example take a picture at that exact time. TRIG is electrically using a differential LVDS receiver, including 100 ohm termination resistors. The rising edge (TRIG_P going from low to high and TRIG_N going from high to low) is the active trigger edge used. Use of TRIG is optional and this port can be left open.

Table 4-5: Trig Interface Pins

| TRIG Pins |  |
| --- | --- |
| TRIG inputs to FARSIGHT | TRIG_P TRIG_N |
| Electrical Signaling standard | TIA/EIA-644 |
| Termination resistance | 100 ohm |
| Minimum TRIG pulse width | 10 ms |

### INTERRUPT INTERFACES

INTERRUPT is an output from FARSIGHT that can be used to alert the host system to an urgent condition. INTERRUPT is electrically using a differential LVDS transmitter. Logic high (INTERRUPT_P high and INTERRUPT_N low) indicates an active interrupt condition. Acknowledging the interrupt condition is documented in the software ICD.

Use of INTERRUPT is optional and this port can be left open. If used, the receiver should be terminated with a 100 termination resistor.

Table 4-6: Interrupt Interface Pins

| TRIG Pins |  |
| --- | --- |
| INTERRUPT outputs from FARSIGHT | INTERRUPT_P INTERRUPT_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Expected termination resistance | 100 ohm |

### GENERAL PURPOSE INTERFACES

For flexibility during the firmware and software development process, a General Purpose Input (GPI) and a General Purpose Output (GPO) have been allocated for use cases that are to be determined. The General Purpose Input (GPI) is a general-purpose input to the FARSIGHT camera. The General Purpose Output (GPO) is a general-purpose out from the FARSIGHT camera.

GPI and GPO are electrically using differential LVDS signaling. The GPI signal includes a 100 ohm termination. If GPO is used, the receiving end should be terminated with 100 ohm.

Connecting to these ports is optional and they can be left open.

Table 4-7: General Purpose Interface Input Parameters

| General Purpose input |  |
| --- | --- |
| General Purpose input to FARSIGHT | GPIN_P GPIN_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Termination resistance | 100 ohm |

Table 4-8: General Purpose Interface Output Parameters

| General Purpose output |  |
| --- | --- |
| General Purpose output from FARSIGHT | GPOUT_P GPOUT_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Expected termination resistance | 100 ohm |

### PROGRAMMING INTERFACE

For the purpose of being able to reprogram the Field Programmable Gate Array (FPGA) inside FARSIGHT, a programming SPI interface is available on the module interface. Programming scheme documented in Software ICD.

Electrically, the four SPI signals are electrically using differential LVDS signaling.

Connecting to these ports is optional and they can be left open. (An open input is pulled to safe state inside Farsight.)

Table 4-9: Programming Interface Input Parameters

| Programming inputs |  |
| --- | --- |
| Programming inputs to FARSIGHT (SPI inputs) | PR_SS_P/PR_SS_N PR_SDI_P/PR_SDI_N PR_SCK_P/PR_SCK_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Termination resistance | 100 ohm |

Table 4-10: Programming Interface Output Parameters

| Programming outputs |  |
| --- | --- |
| Programming output from FARSIGHT (SPI output) | PR_SDO_P PR_SDO_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Expected termination resistance | 100 ohm |

### ETHERNET INTERFACES

This interface provides the primary method of data transfer off the FARSIGHT payload. It operates on the 1000BASE-T standard.

This interface is magnetically isolated and there are two ports provided for redundancy and increased data transfer rate.

Table 4-11: Ethernet Interface Pins

| Ethernet Interfaces |  |
| --- | --- |
| Ethernet 1 pins | ETH1_MX1_P ETH1_MX1_N ETH1_MX2_P ETH1_MX2_N ETH1_MX3_P ETH1_MX3_N ETH1_MX4_P ETH1_MX4_N |
| Ethernet 2 pins | ETH2_MX1_P ETH2_MX1_N ETH2_MX2_P ETH2_MX2_N ETH2_MX3_P ETH2_MX3_N ETH2_MX4_P ETH2_MX4_N |
| Ethernet standard | IEEE 802.3 1000BASE-T |

### PCIe INTERACE

The PCIe interface provides an additional even higher speed data interface for the FARSIGHT payload. FARSIGHT functions as a “PCIe x2 Gen2 Endpoint”.

Two bidirectional “SERDES” (serializer/deserializer) ports (a total of 4 signal pairs) from the FPGA comprise the data links of the PCIe.

An additional SERDES pair is an optional reference clock input to the FPGA.

And furthermore, the PCIe Reset signal from the root complex is also an input to FARSIGHT. PCIE_NRESET is electrically using a differential LVDS receiver, including 100 ohm termination resistor. Logic low (PCIE_NRESET_P low and PCIE_NRESET_N high) indicates an active reset condition.

Table 4-12: High-Speed PCIe Interface Pins

| High-speed PCIe SERDES interfaces |  |
| --- | --- |
| PCIe lane 0, input pins | PCIE0_RX_P PCIE0_RX_N |
| PCIe lane 0, output pins | PCIE0_TX_P PCIE0_TX_N |
| PCIe lane 1, input pins | PCIE1_RX_P PCIE1_RX_N |
| PCIe lane 1, output pins | PCIE1_TX_P PCIE1_TX_N |
| PCIe Reference clock input pins | PCIE_REF_CLK_P PCIE_REF_CLK_N |
| Electrical Signaling standard | “SerDes” TIA/EIA-644 |
| Termination resistance | On-chip termination |

Table 4-13: High-Speed PCIe Interface Pins

| High-speed PCIe SERDES interfaces |  |
| --- | --- |
| PCIe Reset input pins | PCI_NRESET_P PCI_NRESET_N |
| Electrical Signaling standard | LVDS TIA/EIA-644 |
| Termination resistance | 100 ohm |

Since these signals are terminated in the FPGA inside FARSIGHT, it is possible to implement other serial protocols if needed in the future. Some other possible protocols:

- Aurora

- LightFast

- 10GBASE-KR/R

- SATA

### HEATER AND THERMISTOR INTERFACE

FARSIGHT has internal heaters and internal thermistors mounted on the telescope structure and the Focal Plane Printed Circuit Board (PCB). These are all made available for external thermal management from the host vehicle bus.

The Heater and Thermistor parameters for the Focal Plane PCB thermal management are given in table below:

Table 4-14: Focal Plane PCB Heater Interface Pins

| Focal Plane Heater |  |
| --- | --- |
| Pins | IMX_HEATER_P IMX_HEATER_N |
| Voltage Range | 24V to 34V |
| Heater nominal resistance | 990-1010 ohm |
| Location | Focal Plane PCB |

Table 4-15: Focal Plane PCB Thermistor Interface Pins

| Focal Plane Thermistor |  |
| --- | --- |
| Pins | IMX_THERM_P IMX_THERM_N |
| Thermistor | Vishay NTCS0603E3222FMT |

Note: Leads of each heater and each thermistor have resistive bleed-path to ground to allow no-connect these signals.

The heaters and thermistors on the telescope optical assembly

Table 4-16: Optical Assembly Heater Interface Pins

| Heaters Zones 1-6 |  |
| --- | --- |
| Pins | HTRxx_P HTRxx_N |
| Voltage Range | 24V to 34V |
| Heater nominal resistance | 39-157 ohm (depending on channel) |
| Heater zone minimum resistance | >17 ohm |
| Location | Optical Assembly |

Table 4-: Optical Assembly Thermistor Interface Pins

| Optical Assembly Thermistors |  |
| --- | --- |
| Pins | THERMxx_P THERMxx_N |
| Thermistor | Omega TH-44004-xx-T |

## CONNECTOR INTERFACE PINOUT DESCRIPTIONS

This section defines the connector interfaces on FARSIGHT and their pinout.

- J1: Telemetry and Control

- J2: High-speed Serial Data

- J3: Ethernet 1

- J4: Ethernet 2

- J5: Power

- QJ1-6: Optical Assembly Heaters and thermistors

The following subsections define each connector.

### J1: TELEMETRY AND CONTROL

25 pin female Micro-D with jack posts.

(Omnetics MMDS-025-VV-N01 (equivalent), mating example: Omnetics MMDP-025-SS-N02)

Table 4-14: Telemetry and Control Connector Pinout

| Pin | Pin Name | Description |
| --- | --- | --- |
| 1 | RS422_BUS_TO_FARSIGHT_P | RS422 telemetry, input |
| 2 | RS422_BUS_TO_FARSIGHT_N |  |
| 3 | PPS_P | Pulse Per Second timing pulse, LVDS, input |
| 4 | PPS_N |  |
| 5 | 10MHZ_P | 10MHz clock, LVDS, input |
| 6 | 10MHZ_N |  |
| 7 | GPOUT_P | General Purpose Output, LVDS, output |
| 8 | GPOUT_N |  |
| 9 | PR_SS_P | Programming SPI Chip Select, LVDS, input |
| 10 | PR_SS_N |  |
| 11 | PR_SDI_P | Programming SPI MOSI, LVDS, input |
| 12 | PR_SDI_N |  |
| 13 | CHAS | Chassis Shield |
| 14 | RS422_FARSIGHT_TO_BUS_P | RS422 telemetry, output |
| 15 | RS422_FARSIGHT_TO_BUS_N |  |
| 16 | TRIG_P | External Trigger, LVDS, input |
| 17 | TRIG_N |  |
| 18 | INTERRUPT_P | INTERRUPT, LVDS, output |
| 19 | INTERRUPT_N |  |
| 20 | GPIN_P | General Purpose Input, LVDS, input |
| 21 | GPIN_N |  |
| 22 | PR_SDO_P | Programming SPI MISO, LVDS, output |
| 23 | PR_SDO_N |  |
| 24 | PR_SCK_P | Programming SPI clock, LVDS, input |
| 25 | PR_SCK_N |  |

### J2: PCIe SERIAL DATA

26 pin Samtec AcceleRate ARC6 connector.

(Samtec ARF6-08-S-D-A-K-TR, mating example: Samtec ARC6-08-06.0-LU-LD-2-1)

Table 4-15: High-Speed Serial Data Connector Pinout

| Pin Number | Pin Name | Description |  |
| --- | --- | --- | --- |
| 1 | CHAS |  | Chassis Shield |
| 3 | PCIE_REF_CLK_N |  | PCIe Reference Clock Negative, SerDes, input |
| 5 | PCIE_REF_CLK_P |  | PCIe Reference Clock Positive, SerDes, input |
| 7 | CHAS |  | Chassis Shield |
| 9 | PCIE_NRESET_N |  | PCIe nRESET Negative, LVDS, input |
| 11 | PCIE_NRESET_P |  | PCIe nRESET Positive, LVDS, input |
| 13 | CHAS |  | Chassis Shield |
| 15 | PCIE1_RX_P |  | PCIe 1 Positive, SerDes, input |
| 17 | PCIE1_RX_N |  | PCIe 1 Negative, SerDes, input |
| 19 | CHAS |  | Chassis Shield |
| 21 | PCIE0_RX_P |  | PCIe 0 Positive, SerDes, input |
| 23 | PCIE0_RX_N |  | PCIe 0 Negative, SerDes, input |
| 25 | CHAS |  | Chassis Shield |
| 2 | CHAS |  | Chassis Shield |
| 4 | PCIE0_TX_N |  | PCIe 0 Negative, SerDes, output |
| 6 | PCIE0_TX_P |  | PCIe 0 Positive, SerDes, output |
| 8 | CHAS |  | Chassis Shield |
| 10 | PCIE1_TX_N |  | PCIe 1 Negative, SerDes, output |
| 12 | PCIE1_TX_P |  | PCIe 1 Positive, SerDes, output |
| 14 | CHAS |  | Chassis Shield |
| 16 | UNUSED_P |  | Unused |
| 18 | UNUSED_N |  | Unused |
| 20 | CHAS |  | Chassis Shield |
| 22 | UNUSED_P |  | Unused |
| 24 | UNUSED_N |  | Unused |
| 26 | CHAS |  | Chassis Shield |

### J3: Ethernet 1

9 pin male Micro-D.

(Omnetics MMDP-009-VV-N01 (equivalent), mating example: Omnetics MMDS-009-SS-N02)

Table 4-16: Ethernet 1 Connector Pinout

| Pin Number |  | Pin Name | Description |
| --- | --- | --- | --- |
| 1 |  | ETH1_MX4_N | IEEE 802.3, MX4- |
|  | 6 | ETH1_MX4_P | IEEE 802.3, MX4+ |
| 2 |  | ETH1_MX3_N | IEEE 802.3, MX3- |
|  | 7 | ETH1_MX3_P | IEEE 802.3, MX3+ |
| 3 |  | SHLD | Chassis Shield |
|  | 8 | ETH1_MX2_N | IEEE 802.3, MX2- |
| 4 |  | ETH1_MX2_P | IEEE 802.3, MX2+ |
|  | 9 | ETH1_MX1_N | IEEE 802.3, MX1- |
| 5 |  | ETH1_MX1_P | IEEE 802.3, MX1+ |

### J4: Ethernet 2

9 pin male Micro-D.

(Omnetics MMDP-009-VV-N01 (equivalent), mating example: Omnetics MMDS-009-SS-N02)

Table 4-17: Ethernet 2 Connector Pinout

| Pin Number |  | Pin Name | Description |
| --- | --- | --- | --- |
| 1 |  | ETH2_MX4_N | IEEE 802.3, MX4- |
|  | 6 | ETH2_MX4_P | IEEE 802.3, MX4+ |
| 2 |  | ETH2_MX3_N | IEEE 802.3, MX3- |
|  | 7 | ETH2_MX3_P | IEEE 802.3, MX3+ |
| 3 |  | SHLD | Shield |
|  | 8 | ETH2_MX2_N | IEEE 802.3, MX2- |
| 4 |  | ETH2_MX2_P | IEEE 802.3, MX2+ |
|  | 9 | ETH2_MX1_N | IEEE 802.3, MX1- |
| 5 |  | ETH2_MX1_P | IEEE 802.3, MX1+ |

### J5: POWER

9 pin female Micro-D.

(Omnetics MMDS-09-VV-N01, mating example: Omnetics MMDP-009-SS-N02)

Table 4-18: Power Connector Pinout

| Pin Number | Pin Name | Description |
| --- | --- | --- |
| 1 | PWR | 28V power |
| 2 | PWR |  |
| 3 | CHAS | Chassis |
| 4 | IMX_THERM_N | Focal Plane PCB Thermistor |
| 5 | IMX_THERM_P |  |
| 6 | PGND | Power return |
| 7 | PGND |  |
| 8 | IMX_HEATER_N | Focal Plane PCB Heater |
| 9 | IMX_HEATER_P |  |

### QJ1 -QJ6: OTA HEATERS AND THERMISTORS

Heaters and thermistors on the optical assembly are physically located on the side of the optical assembly on a panel:

Each connector is a heater zone with corresponding thermistor(s) for that zone. All heaters in a zone are to be controlled as a single heater (all turn on or off at the same time).

- THERMAL MANAGEMENT

The heater and thermistor interface are  meant for the satellite bus to be able to perform thermal management of the FARSIGHT payload. This is the case for both survival (FARSIGHT powered off) and operational (FARSIGHT powered on and operating) temperatures for FARSIGHT.

Except possibly being able to deliver additional temperature sensor readings to the bus, FARSIGHT itself does not perform any thermal management.

Table below shows thermal management set points that will inform the satellite bus thermal management scheme.

| Heater Zone | Sensor 1 | Sensor 2 | Sensor 3 | Proportional control % | On Set Point (°C) | Off Set Point (°C) | Critical Set Point (°C) | Sensor Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Farsight_Therm1 | Farsight_Therm2 | Farsight_Therm13 | 100 | 10 | 11 | 50 | TH-44004-80-T |
| 2 | Farsight_Therm3 | Farsight_Therm4 |  | 100 | 10 | 11 | 50 | TH-44004-80-T |
| 3 | Farsight_Therm5 | Farsight_Therm6 |  | 100 | 10 | 11 | 50 | TH-44004-80-T |
| 4 | Farsight_Therm7 | Farsight_Therm8 |  | 100 | 10 | 11 | 50 | TH-44004-80-T |
| 5 | Farsight_Therm9 | Farsight_Therm10 |  | 100 | 10 | 11 | 50 | TH-44004-80-T |
| 6 | Farsight_Therm11 | Farsight_Therm12 |  | 100 | 10 | 11 | 50 | TH-44004-80-T |
| 7 | Therm14 |  |  |  |  |  |  | 0118MF |

# GROUNDING

This section defines the grounding scheme employed in FARSIGHT.

FARSIGHT chassis/shield is isolated from signal ground. Power return is connected to signal ground.

All interfaces between FARSIGHT and the host space vehicle are using differential signaling so signal ground is not available on any control interface connectors. This reduces the risk of unintentional ground paths through signal harnesses.

The remote heater and thermistor interface provided on the FARSIGHT Sensor Board are accessible through the avionics board but are not connected to either FARSIGHT P/SGND or chassis.

FARSIGHT electrical ground is called signal and power ground, SGND and PGND. These are all the same, but PGND is meant to provide lowest impedance power ground return. Any SGND pin on an interface is meant to be signal return path for surrounding signals.

# INTERNAL ELECTRICAL INTERFACE DEFINITIONS

There are a few electrical interfaces that exist inside of  FARSIGHT. These include the Avionics board to the Sensor board as well as the Avionics board to the focusing mechanism. These interfaces, along with harness design guidelines are detailed below.

## J6: Focusing Mechanism Interface

19 pin female Glenair Series 79 with jack posts.

(Glenair 790-044SE-19MEPA, mating example: Glenair 790-045PE-19MM)

Table 7-1: Focusing Mechanism Interface Pinout

| Pin Number |  | Pin Name | Description | Twisted Pair | Functional Group |
| --- | --- | --- | --- | --- | --- |
| 1 |  | LVDT_S_P | Secondary 1 winding + (red wire) | Pair 0 | LVDT position sensor |
|  | 11 | LVDT_S_N | Secondary 0 winding + (black wire) |  |  |
|  |  |  | Secondary 0 winding – (blue wire) | Shorted at LVDT |  |
|  |  |  | Secondary 1 winding – (green wire) |  |  |
| 2 |  | LVDT_S_SHIELD | Shield for secondary winding (drain wire) |  |  |
|  | 12 | LVDT_P_SHIELD | Shield for primary winding (drain wire) |  |  |
| 3 |  | LVDT_P_P | Primary winding + (yellow wire) | Pair 1 |  |
|  | 13 | LVDT_P_N | Primary winding – (brown wire) |  |  |
| 4 |  | GND | No connect in harness |  | NC |
|  | 14 | GND | No connect in harness |  |  |
| 5 |  | GND | No connect in harness |  |  |
|  | 15 | GND | No connect in harness |  |  |
| 6 |  | COIL_SHIELD | Shield for stepper motor harness |  | Stepper motor |
|  | 16 | SEC_COILB_P | Stepper motor secondary coil B + | Pair 4 |  |
| 7 |  | PRIM_COILA_P | Stepper motor primary coil A + | Pair 3 |  |
|  | 17 | SEC_COILB_N | Stepper motor secondary coil B - | Pair 4 |  |
| 8 |  | PRIM_COILA_N | Stepper motor primary coil A - | Pair 3 |  |
|  | 18 | SEC_COILA_N | Stepper motor secondary coil A - | Pair 2 |  |
| 9 |  | PRIM_COILB_N | Stepper motor primary coil B - | Pair 5 |  |
|  | 19 | SEC_COILA_P | Stepper motor secondary coil A+ | Pair 2 |  |
| 10 |  | PRIM_COILB_P | Stepper motor primary coil B + | Pair 5 |  |

## Focusing Mechanism Harness

The harness between J6 and the LVDT and stepper motor is illustrated below in Figure 7-1.

It contains multiple shields for protecting the sensitive LVDT circuitry from the noise in the environment and that generated by the motor driver/coils.

LVDT Secondary negative wires (blue and green wires) should be shorted close to the LVDT housing.

For this purpose the primary and secondary coils of the LVDT are individually shielded and terminated with drain wires to shielded pins in the harness connector, and to grounding lugs near the LVDT housing.

The motor coils also have a shield which is similarly tied off.

To provide complete shield coverage at the harness connector, a short overshield is specified which attaches to the backshell of the connector and makes contact to the 3 shield bundles inside of it, covering the exposed region of the wires and providing a low impedance path to the connector shell.

Figure 7-1: Focusing mechanism harness illustration

## J7: Sensor Board Interface

69 pin male Omnetics Nano-D High speed connector.

(Omnetics A49256-001, mating harness provided by Turion)

Table 7-2: Sensor Board Interface Pinout

| Pin Number |  | Pin Name | Description |
| --- | --- | --- | --- |
| 1 |  | GND | SLVS-EC Pair 6 Shield |
| 2 |  | SLVS_EC_6_P | SLVS-EC Pair 6 + |
| 3 |  | SLVS_EC_6_N | SLVS-EC Pair 6 - |
|  | 36 | SLVS_EC_7_P | SLVS-EC Pair 7 + |
|  | 37 | SLVS_EC_7_N | SLVS-EC Pair 7 - |
|  | 38 | GND | SLVS-EC Pair 7 Shield |
| 5 |  | GND | SLVS-EC Pair 4 Shield |
| 6 |  | SLVS_EC_4_P | SLVS-EC Pair 4 + |
| 7 |  | SLVS_EC_4_N | SLVS-EC Pair 4 - |
|  | 40 | SLVS_EC_5_P | SLVS-EC Pair 5 + |
|  | 41 | SLVS_EC_5_N | SLVS-EC Pair 5 - |
|  | 42 | GND | SLVS-EC Pair 5 Shield |
| 9 |  | GND | SLVS-EC Pair 2 Shield |
| 10 |  | SLVS_EC_2_P | SLVS-EC Pair 2 + |
| 11 |  | SLVS_EC_2_N | SLVS-EC Pair 2 - |
|  | 44 | SLVS_EC_3_P | SLVS-EC Pair 3 + |
|  | 45 | SLVS_EC_3_N | SLVS-EC Pair 3 - |
|  | 46 | GND | SLVS-EC Pair 3 Shield |
| 13 |  | GND | SLVS-EC Pair 0 Shield |
| 14 |  | SLVS_EC_0_P | SLVS-EC Pair 0 + |
| 15 |  | SLVS_EC_0_N | SLVS-EC Pair 0 - |
|  | 48 | SLVS_EC_1_P | SLVS-EC Pair 1 + |
|  | 49 | SLVS_EC_1_N | SLVS-EC Pair 1 - |
|  | 50 | GND | SLVS-EC Pair 1 Shield |
| 17 |  | GND | Ground |
|  | 52 | 1V8_IMX | 1.8V supply |
| 18 |  | GND | Ground |
|  | 53 | 1V1_IMX | 1.1V supply |
| 19 |  | GND | Ground |
|  | 54 | 1V1_IMX | 1.1V supply |
| 20 |  | GND | Ground |
|  | 55 | 1V1_IMX_SNS | 1.1V sense |
| 21 |  | GND | Ground |
|  | 56 | 4V0_REG | 4V supply |
| 22 |  | GND | Ground |
|  | 57 | 4V0_REG | 4V supply |
| 23 |  | GND | Ground |
|  | 58 | 3V3_IMX_SNS | 3.3V sense |
| 24 |  | GND | Ground |
|  | 59 | 2V9_IMX_SNS | 2.9V sense |
| 25 |  | IMX_2V9_EN_BUF | 2.9V power enable |
|  | 60 | IMX_3V3_EN_BUF | 3.3V power enable |
| 26 |  | IMX_HEATER_N | Sensor board heater - |
|  | 61 | IMX_HEATER_P | Sensor board heater + |
| 27 |  | GND | Ground |
|  | 62 | IMX_SPI_XCE_BUF | SPI CE |
| 28 |  | GND | Ground |
|  | 63 | IMX_SPI_SDO | SPI SDO |
| 29 |  | GND | Ground |
|  | 64 | IMX_SPI_SCK_BUF | SPI SCK |
| 30 |  | GND | Ground |
|  | 65 | IMX_SPI_SDI_BUF | SPI SDI |
| 31 |  | GND | Ground |
|  | 66 | IMX_XCLR_BUF | Sensor CLR |
| 32 |  | GND | Ground |
|  | 67 | IMX_XTRIG1_BUF | Sensor TRIG1 |
| 33 |  | GND | Ground |
|  | 68 | IMX_TOUT0 | Sensor TOUT0 |
| 34 |  | IMX_THERM_N | Sensor Board Thermistor - |
|  | 69 | IMX_THERM_P | Sensor Board Thermistor + |
| 35 |  | IMX_OSC_EN_BUF | Sensor Board Oscillator Enable |